Error: Reference source not found

Table of Contents

Overview	6

Driver Categories	6

Hardware and Application Requirements	6

Document Version	6

Open/Close driver VI	7

GL500_Open	7

GL500_Close	7

Middle-level drivers (group VI)	8

Overview of middle-level drivers	8

GL500_AMP_Config.vi	8

GL500_Alarm_Config.vi	9

GL500_Control_Config.vi	10

GL500_Event_Config.vi	10

GL500_IF_Config.vi	11

GL500_Information_Config.vi	11

GL500_Measure_Config.vi	11

GL500_Option_Config.vi	12

GL500_Pulse_Config.vi	12

GL500_Status_Config.vi	13

GL500_Transfer_Config.vi	13

GL500_Data_Config.vi	14

GL500_ETrigger_Config.vi	14

Higher-level (command VI)	16

Overview of higher-level drivers	16

GL500_Alarm_Channel_Output.vi	16

GL500_Alarm_Channel_Set.vi	17

GL500_Alarm_Combination.vi	17

GL500_Alarm_Logic_Output.vi	18

GL500_Alarm_Logic_Set.vi	18

GL500_Alarm_Pulse_Output.vi	18

GL500_Alarm_Pulse_Set.vi	19

GL500_Amp_Azero.vi	19

GL500_Amp_Filter.vi	20

GL500_Amp_Input.vi	20

GL500_Amp_Range.vi	20

GL500_Amp_Scan.vi	21

GL500_Amp_Type.vi	21

GL500_Annotation_Channel.vi	21

GL500_CLS.vi	22

GL500_Control_Busy.vi	22

GL500_Control_Master.vi	22

GL500_Control_Rec.vi	22

GL500_Control_Time.vi	23

GL500_Data_Capture.vi	23

GL500_Data_Kind.vi	23

GL500_Data_Sample.vi	24

GL500_ESE.vi	24

GL500_ESR.vi	24

GL500_ETrigger_CHA.vi	25

GL500_ETrigger_CHB.vi	25

GL500_ETrigger_Combination.vi	26

GL500_ETrigger_Logic.vi	26

GL500_ETrigger_Pretrigger.vi	27

GL500_ETrigger_Pulse.vi	27

GL500_ETrigger_Source.vi	28

GL500_ETrigger_Stop.vi	28

GL500_Event_Block.vi	28

GL500_Event_Function.vi	29

GL500_Event_Kind.vi	29

GL500_Event_Over.vi	29

GL500_Event_Sample.vi	30

GL500_File_Auto.vi	30

GL500_File_CD.vi	30

GL500_File_CP.vi	31

GL500_File_List.vi	31

GL500_File_Load.vi	31

GL500_File_MD.vi	31

GL500_File_MV.vi	32

GL500_File_RD.vi	32

GL500_File_RM.vi	32

GL500_File_Save.vi	32

GL500_File_Space.vi	32

GL500_IDN.vi	33

GL500_IF_Echo.vi	33

GL500_IF_ID.vi	33

GL500_IF_IP.vi	34

GL500_IF_Mac.vi	34

GL500_IF_Nlcode.vi	34

GL500_IF_Port.vi	35

GL500_IF_SubNet.vi	35

GL500_LP_ChFilter.vi	35

GL500_LP_ChFunction.vi	36

GL500_LP_ChRange.vi	36

GL500_LP_ChSlope.vi	37

GL500_LP_Function.vi	37

GL500_Mac_Address.vi	37

GL500_Mac_Execute.vi	38

GL500_Meas_Output_Ack.vi	38

GL500_Meas_Output_Clr.vi	38

GL500_Meas_Output_Head.vi	38

GL500_Meas_Output_One.vi	39

GL500_Meas_Output_Point.vi	39

GL500_Meas_Output_Stat.vi	39

GL500_Meas_Output_Stop.vi	39

GL500_Meas_Size.vi	40

GL500_Meas_Start.vi	40

GL500_Meas_Stop.vi	40

GL500_Meas_Time.vi	40

GL500_OPT.vi	40

GL500_Option_Date.vi	41

GL500_Option_Initial.vi	41

GL500_Option_Pow.vi	41

GL500_Option_Temp.vi	41

GL500_Option_Tunit.vi	42

GL500_SAV.vi	42

GL500_SRE.vi	42

GL500_STB.vi	42

GL500_Status_Condition.vi	43

GL500_Status_EESE.vi	43

GL500_Status_EESR.vi	43

GL500_Status_Error.vi	43

GL500_Status_Filter.vi	43

GL500_Status_Lock.vi	44

GL500_Status_Remote.vi	44

GL500_Transfer_Close.vi	44

GL500_Transfer_Data.vi	44

GL500_Transfer_Head.vi	45

GL500_Transfer_ID.vi	45

GL500_Transfer_Open.vi	45

GL500_Transfer_Source.vi	45

GL500_Trigger_Channel.vi	46

GL500_Trigger_Combination.vi	46

GL500_Trigger_Logic.vi	47

GL500_Trigger_Pulse.vi	47

GL500_Trigger_Source.vi	48

GL500_Up_Execute.vi	48

# Overview

This program allows you to operate the GL500 Series, as well as change settings and read out data, from a computer through a TCP/IP or USB connection.

Programs are categorized in two VIs based on their functions.

Prerequisite

The following requirements must be met to use this program.

- Experience in program development using LabVIEW.

## Driver Categories

VI drivers are categorized as either higher-level or middle-level.

Higher-level VIs:	A single VI corresponds to a single command.

Middle-level VIs:	A single VI executes multiple commands within a group.

# Hardware and Application Requirements

| CPU | Pentium III 600MHz or higher recommended |
| --- | --- |
| OS | Windows 2000 or XP recommended |
| Memory | 128M or higher recommended |
| Screen resolution | 1024 * 768 or higher recommended |
| Application | LabVIEW 6.1 or higher |

# Document Version

| Version | Description | Revision Date |
| --- | --- | --- |
| 1.00 | First edition | 2005/02/28 |

# Open/Close driver VI

## GL500_Open

List of VIs used

GL500_ Open.vi

Establishes a connection with GL500 to allow control from LabVIEW.

Switching between two VI types can be achieved from "Select Type" in the VI pop-up menu.

Input values

| address | Str | (TCP/IP) Inputs TCP/IP address Format: "xxx.xxx.xxx.xxx" |
| --- | --- | --- |
| remote port | U16 | (TCP/IP) Port number |
| ID | I32 | (USB) USB ID number |

Output values

| Connect ID out | I32, TCP ID | Connected device number: outputs connection ID (TCP/IP) or device number (USB) |
| --- | --- | --- |
| error out | error cluster | Outputs error code |

## GL500_Close

List of VIs used

GL500_Close.vi

Disconnects GL500 from your PC.

Input values

| Connect ID | I32, TCP ID | Connected device number: inputs connection ID (TCP/IP) or device number (USB) |
| --- | --- | --- |
| error in | error cluster | Inputs error code |

Output values

| error out | error cluster | Outputs error code |
| --- | --- | --- |

# Middle-level drivers (group VI)

## Overview of middle-level drivers

- A middle-level VI driver is a collection of GL500 commands within a group.

Common input and output terminal configurations

Input values

| Connect ID | I32, TCP ID | Connected device number: inputs connection ID (TCP/IP) or device number (USB) |
| --- | --- | --- |
| mask | cluster bool | Selects commands to be disregarded in setting items Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| error in | error cluster | Inputs error code |

Output values

| Connect ID out | I32, TCP ID | Connected device number: copy of input ID Outputs connection ID(TCP/IP) or device number (USB) |
| --- | --- | --- |
| error out | error cluster | Outputs error code |

## GL500_AMP_Config.vi

Input values

| CH | I32 | Specifies channel |
| --- | --- | --- |
| Set | cluster | Sends the setting commands in Amp group |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns setting status |

Output values

| Query out | cluster | Receives Amp group settings |
| --- | --- | --- |

## GL500_Alarm_Config.vi

Input values

| CH | I32 | Specifies channel |
| --- | --- | --- |
| Channel alarm | cluster | Sends Alarm Amp CH settings command |
| Pulse alarm | cluster | Sends Alarm Pulse settings command |
| Logic alarm | cluster | Sends Alarm Logic settings command |
| Trigger | cluster | Sends Alarm Trigger settings command |
| Combination | U16 | Sends Alarm combination settings command |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Channel alarm out | cluster | Receives Alarm Amp CH settings |
| --- | --- | --- |
| Pulse alarm out | cluster | Receives Alarm Pulse settings |
| Logic alarm out | cluster | Receives Alarm Logic settings |
| Trigger out | cluster | Receives Alarm Trigger settings |
| Combination out | U16 | Receives Alarm combination |

## GL500_Control_Config.vi

Input values

| Set | cluster | Sends Control settings command |
| --- | --- | --- |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives Control settings |
| --- | --- | --- |

## GL500_Event_Config.vi

Input values

| Set | cluster | Sends Event settings command |
| --- | --- | --- |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives Event settings |
| --- | --- | --- |

## GL500_IF_Config.vi

Input values

| Set | cluster | Sends IF settings command |
| --- | --- | --- |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives IF settings |
| --- | --- | --- |

## GL500_Information_Config.vi

Output values

| Info | cluster | Receives Information settings |
| --- | --- | --- |

## GL500_Measure_Config.vi

Input values

| Set | cluster | Sends Measure settings command |
| --- | --- | --- |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives Measure settings |
| --- | --- | --- |

## GL500_Option_Config.vi

Input values

| Set | cluster | Sends Option settings command |
| --- | --- | --- |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives Option settings |
| --- | --- | --- |

## GL500_Pulse_Config.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Set | cluster | Sends Pulse settings command |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives Pulse settings |
| --- | --- | --- |

## GL500_Status_Config.vi

Input values

| Filter No. | I32 | Specifies filter location |
| --- | --- | --- |
| Set | cluster | Sends Status settings command |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives Status settings |
| --- | --- | --- |

## GL500_Transfer_Config.vi

Input values

| Set | cluster | Sends Transfer settings command |
| --- | --- | --- |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives Transfer settings |
| --- | --- | --- |

## GL500_Data_Config.vi

Input values

| Set | cluster | Sends Data settings command |
| --- | --- | --- |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| Query out | cluster | Receives Data settings |
| --- | --- | --- |

## GL500_ETrigger_Config.vi

Input values

| CHA | cluster | Sends CHA settings command |
| --- | --- | --- |
| CHB | cluster | Sends CHB settings command |
| Pulse | cluster | Sends Pulse settings command |
| Logic | U16 array | Sends Logic settings command |
| Source | cluster | Sends Source settings command |
| COND | U16 | Sends event Start/Stop settings command |
| Mask | cluster bool | Selects settings commands to be disregarded when sending/receiving data Disregards settings that are not changed. This reduces the number of commands sent, increasing process speed. |
| Query | bool | Returns settings status |

Output values

| CHA out | cluster | Receives CHA settings |
| --- | --- | --- |
| CHB out | cluster | Receives CHB settings |
| Pulse out | cluster | Receives Pulse settings |
| Logic out | U16 array | Receives Logic settings |
| Source out | cluster | Receives Source settings |

# Higher-level (command VI)

## Overview of higher-level drivers

- A single higher-level VI driver corresponds to a single GL500 command.

Common input and output terminal configurations

Input values

| Connect ID | I32, TCP ID | Connected device number: inputs connection ID (TCP/IP) or device number (USB) |
| --- | --- | --- |
| error in | error cluster | Inputs error code |

Output values

| Connect ID out | I32, TCP ID | Connected device number: copy of input ID Outputs connection ID(TCP/IP) or device number (USB) |
| --- | --- | --- |
| error out | error cluster | Outputs error code |

## GL500_Alarm_Channel_Output.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Range | U16 | Inputs alarm number |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Alarm_Channel_Set.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Trigger | Select (U16) | OFF, High, Low, Window |
| Window | Select (U16) | IN/OUT/WinIn/WinOut |
| High/Low | String | Value when IN/OUT |
| Window Min | String | Lower limit when WinIn/WinOut |
| Window Max | String | Upper limit when WinIn/WinOut |
| Query | bool | Returns settings status |

Output values

| Trigger | I32 | OFF, High, Low, Window |
| --- | --- | --- |
| Window | Select (U16) | IN/OUT/WinIn/WinOut |
| High/Low | String | Value when IN/OUT |
| Window Min | String | Lower limit when WinIn/WinOut |
| Window Max | String | Upper limit when WinIn/WinOut |

## GL500_Alarm_Combination.vi

Input values

| Condition | I32 | AND/OR |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Alarm_Logic_Output.vi

Input values

| Logic no. | I32 | Specifies channel |
| --- | --- | --- |
| Alarm no. | I32 | Specifies alarm number |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Alarm_Logic_Set.vi

Input values

| Logic no. | I32 | Specifies channel |
| --- | --- | --- |
| Set | Select (U16) | OFF/HI/LO |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Alarm_Pulse_Output.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Alarm no. | Select (U16) | Specifies alarm number |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Alarm_Pulse_Set.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| SET | Select (U16) | TYPE OFF, High, Low, Window |
|  | Select (U16) | Window IN/OUT/WinIn/WinOut |
|  | String | High/Low Value when IN/OUT |
|  | String | Window Min Lower limit when WinIn/WinOut |
|  | String | Window Max Upper limit when WinIn/WinOut |
| Query | bool | Returns settings status |

Output values

| Query Out | I32 | TYPE OFF, High, Low, Window |
| --- | --- | --- |
|  | Select (U16) | Window IN/OUT/WinIn/WinOut |
|  | String | High/Low Value when IN/OUT |
|  | String | Window Min Lower limit when WinIn/WinOut |
|  | String | Window Max Upper limit when WinIn/WinOut |

## GL500_Amp_Azero.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Range | Select (U16) | Selects zero setting ON/OFF |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Amp_Filter.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Range | Select (U16) | Selects filter OFF, LINE, 5Hz, 50Hz, 500Hz |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Amp_Input.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Range | Select (U16) | Selects input OFF, DC, TEMP, GND |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Amp_Range.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Range | Select (U16) | Selects range 100mV, 200mV, 500mV, 1V, 5V, 10V, 20V, 50V, 100V TCK, TCJ, TCT, TCR, TCE, TCB, TCS, TCN, TCW |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Amp_Scan.vi

Input values

| Scan | Select (U16) | Selects number of input channels |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Amp_Type.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Range | Select (U16) | Selects range 100mV, 200mV, 500mV, 1V, 5V, 10V, 20V, 50V, 100V TCK, TCJ, TCT, TCR, TCE, TCB, TCS, TCN, TCW |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Module type Returns VF, MF, or MS |
| --- | --- | --- |

## GL500_Annotation_Channel.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Range | String | Inputs annotation |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_CLS.vi

Clears memory buffer

## GL500_Control_Busy.vi

Output values

| Query out | I32 | Confirms whether synchronous start is ready |
| --- | --- | --- |

## GL500_Control_Master.vi

Output values

| Query out | I32 | Master/Slave query |
| --- | --- | --- |

## GL500_Control_Rec.vi

Input values

| Rec | I32 | START, STOP |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Control_Time.vi

Input values

| Time | cluster | [STR] start time [STR] stop time [STR] trigger time |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Time out | cluster | [STR] start time [STR] stop time [STR] trigger time |
| --- | --- | --- |

## GL500_Data_Capture.vi

Input values

| Capture | Select (U16) | DISK/MEM |
| --- | --- | --- |
| File path | String | File path |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Data_Kind.vi

Input values

| Query | bool | Returns settings status |
| --- | --- | --- |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |
| Memory data | Select (U16) | Memory data capturing information |

## GL500_Data_Sample.vi

Input values

| Range | Select (U16) | Sets sampling rate 1/2/5/10/20/50/100/200/500(MS) 1/2/5/10/20/30/60/120/300/600/1200/1800(S) |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_ESE.vi

Input values

| ESE | I32 | Sets ESE |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | ESE return value |
| --- | --- | --- |

## GL500_ESR.vi

Input values

| ESR | I32 | Sets ESR |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | ESR return value |
| --- | --- | --- |

## GL500_ETrigger_CHA.vi

Input values

| Start/Stop | I32 | Selects start/stop |
| --- | --- | --- |
| CH | I32 | Selects channel |
| SET | Select (U16) | TYPE OFF, High, Low, Window |
|  | Select (U16) | Window IN/OUT/WinIn/WinOut |
|  | String | High/Low Value when IN/OUT |
|  | String | Window Min Lower limit when WinIn/WinOut |
|  | String | Window Max Upper limit when WinIn/WinOut |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | TYPE OFF, High, Low, Window |
| --- | --- | --- |
|  | Select (U16) | Window IN/OUT/WinIn/WinOut |
|  | String | High/Low Value when IN/OUT |
|  | String | Window Min Lower limit when WinIn/WinOut |
|  | String | Window Max Upper limit when WinIn/WinOut |

## GL500_ETrigger_CHB.vi

Input values

| Start/Stop | I32 | Selects start/stop |
| --- | --- | --- |
| CH | I32 | Selects channel |
| SET | Select (U16) | TYPE OFF, High, Low, Window |
|  | Select (U16) | Window IN/OUT/WinIn/WinOut |
|  | String | High/Low Value when IN/OUT |
|  | String | Window Min Lower limit when WinIn/WinOut |
|  | String | Window Max Upper limit when WinIn/WinOut |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | TYPE OFF, High, Low, Window |
| --- | --- | --- |
|  | Select (U16) | Window IN/OUT/WinIn/WinOut |
|  | String | High/Low Value when IN/OUT |
|  | String | Window Min Lower limit when WinIn/WinOut |
|  | String | Window Max Upper limit when WinIn/WinOut |

## GL500_ETrigger_Combination.vi

Input values

| Start/Stop | I32 | Selects start/stop |
| --- | --- | --- |
| Combination | Select (U16) | Sets combination |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_ETrigger_Logic.vi

Input values

| Start/Stop | I32 | Selects start/stop |
| --- | --- | --- |
| Channel | I32 | Specifies channel |
| Range | Select (U16) | Sets event trigger type OFF, HI, LO |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_ETrigger_Pretrigger.vi

Input values

| Pretrigger | Select (U16) | Sets pretrigger |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_ETrigger_Pulse.vi

Input values

| Start/Stop | I32 | Selects start/stop |
| --- | --- | --- |
| CH | I32 | Selects channel |
| SET | Select (U16) | TYPE OFF, High, Low, Window |
|  | Select (U16) | Window IN/OUT/WinIn/WinOut |
|  | String | High/Low Value when IN/OUT |
|  | String | Window Min Lower limit when WinIn/WinOut |
|  | String | Window Max Upper limit when WinIn/WinOut |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | TYPE OFF, High, Low, Window |
| --- | --- | --- |
|  | Select (U16) | Window IN/OUT/WinIn/WinOut |
|  | String | High/Low Value when IN/OUT |
|  | String | Window Min Lower limit when WinIn/WinOut |
|  | String | Window Max Upper limit when WinIn/WinOut |

## GL500_ETrigger_Source.vi

Input values

| Start/Stop | I32 | Selects start/stop |
| --- | --- | --- |
| Source | Select (U16) | Sets event source OFF, AMP, EXTER |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_ETrigger_Stop.vi

Input values

| STOP | Select (U16) | Specifies trigger stop ON/OFF |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Event_Block.vi

Input values

| Block | Select (U16) | Specifies event trigger memory block 1,2,4,8,16 |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Event_Function.vi

Input values

| Function | Select (U16) | Selects event ON/OFF |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Event_Kind.vi

Output values

| Query out | Select (U16) | Returns event memory status |
| --- | --- | --- |

## GL500_Event_Over.vi

Input values

| Event Over | Select (U16) | Sets event repetition ON/OFF |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Event_Sample.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Sample | Select (U16) | Selects event sampling rate 2, 4, 5, 8, 10, 20, 40, 50, 100, 200, 500 (US) 1, 2, 5, 10, 20, 50, 100, 200, 500 (MS) 1 (S) |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_File_Auto.vi

Input values

| Auto | Select (U16) | Sets automatic file save |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_File_CD.vi

Input values

| Folder | String | Specifies channel |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | String | Returns current location of folder |
| --- | --- | --- |

## GL500_File_CP.vi

Input values

| Path | String | File name to be copied (source) |
| --- | --- | --- |
| Copy Path | String | Copy destination file name |

## GL500_File_List.vi

Input values

| Format | Select (U16) | Specifies file list format Short/Long |
| --- | --- | --- |
| Filter | String | File name filter |
| Query | bool | Returns settings status |

Output values

| List | String array | Returns file list |
| --- | --- | --- |

## GL500_File_Load.vi

Input values

| Load | String | Specifies settings file to read in |
| --- | --- | --- |

## GL500_File_MD.vi

Input values

| Folder | Select (U16) | Specifies folder to create |
| --- | --- | --- |

## GL500_File_MV.vi

Input values

| File | String | Original file name |
| --- | --- | --- |
| ReName File | String | Renamed file name |

## GL500_File_RD.vi

Input values

| Path | String | Deleting folder name |
| --- | --- | --- |

## GL500_File_RM.vi

Input values

| File | String | Deleting file name |
| --- | --- | --- |

## GL500_File_Save.vi

Input values

| Path | String | Specifies settings file name to save |
| --- | --- | --- |

## GL500_File_Space.vi

Output values

| Space | Double | Open space in PC card |
| --- | --- | --- |

## GL500_IDN.vi

Output values

| Corporation | STR | GL500 manufacturer name |
| --- | --- | --- |
| Machines | STR | GL500 model type |
| SerialNo | STR | GL500 serial number |
| Firmware | STR | GL500 firmware version |

## GL500_IF_Echo.vi

Input values

| Echo | Select (U16) | Sets echo ON/OFF |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_IF_ID.vi

Input values

| ID | I32 | Specifies USB device ID |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | Returns the current settings |
| --- | --- | --- |

## GL500_IF_IP.vi

Input values

| IP | String | Specifies IP address |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | String | Returns the current settings |
| --- | --- | --- |

## GL500_IF_Mac.vi

Output values

| Mac address | Select (U16) | Returns MAC Address of device |
| --- | --- | --- |

## GL500_IF_Nlcode.vi

Input values

| Nlcode | Select (U16) | Specifies line feed code CR_LF/CR/LF |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_IF_Port.vi

Input values

| Port | I32 | Specifies port number |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | Returns the current settings |
| --- | --- | --- |

## GL500_IF_SubNet.vi

Input values

| SubNet | String | Specifies subnet mask |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | String | Returns the current settings |
| --- | --- | --- |

## GL500_LP_ChFilter.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Filter | Select (U16) | Specifies filter OFF/ON |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_LP_ChFunction.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Function | Select (U16) | Specifies pulse function OFF/COUNT/REVO/INST |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_LP_ChRange.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Range | Select (U16) | Selects pulse range 5/50/500/(C)5/50(KC) |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_LP_ChSlope.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Slope | Select (U16) | Selects pulse slope HI/LO |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_LP_Function.vi

Input values

| Function | Select (U16) | Selects logic/pulse OFF/LOGI/PUL |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Mac_Address.vi

Input values

| Address | String | Specifies MAC Address of device |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Mac_Execute.vi

Writes to EPROM.

## GL500_Meas_Output_Ack.vi

Input values

| Channel | I32 | Specifies channel |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Data String | String | Data transfer (string) |
| --- | --- | --- |
| Data | [I16] | Data transfer (binary) |
| Byte | I32 | Number of transferred data |

## GL500_Meas_Output_Clr.vi

Output values

| Query out | String | Clears data transfer buffer |
| --- | --- | --- |

## GL500_Meas_Output_Head.vi

Output values

| Header | String | Returns header information |
| --- | --- | --- |

## GL500_Meas_Output_One.vi

Output values

| Data | [I16] | Transfers data in one record (binary) |
| --- | --- | --- |
| Byte | I32 | Number of transferred data |

## GL500_Meas_Output_Point.vi

Input values

| Point | I32 | Specifies number of data buffers |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | Returns the current settings |
| --- | --- | --- |

## GL500_Meas_Output_Stat.vi

Output values

| BufferSize | I32 | Number of data in buffer |
| --- | --- | --- |
| DataNo | I32 | Data number |
| Break | I32 | Number of data discarded |

## GL500_Meas_Output_Stop.vi

Aborts data transfer.

## GL500_Meas_Size.vi

Output values

| Size | Select (U16) | Returns number of data sent |
| --- | --- | --- |

## GL500_Meas_Start.vi

Starts data capturing.

## GL500_Meas_Stop.vi

Ends data capturing.

## GL500_Meas_Time.vi

Output values

| Start Time | String | Returns measurement start time |
| --- | --- | --- |
| Stop Time | String | Returns measurement end time |

## GL500_OPT.vi

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |
| Option | [STR] | Returns list of optional devices |

## GL500_Option_Date.vi

Input values

| Date | STR | Inputs date |
| --- | --- | --- |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Option_Initial.vi

Initializes device settings.

## GL500_Option_Pow.vi

Input values

| Pow | Select (U16) | Sets power ON start |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Option_Temp.vi

Input values

| Temp | Select (U16) | Sets ambient temperature correction |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Option_Tunit.vi

Input values

| Tunit | Select (U16) | Switches display of Celsius/Fahrenheit CELS/FAHR |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_SAV.vi

Writes in settings.

## GL500_SRE.vi

Input values

| SRE | Select (U16) | Sets service request query |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_STB.vi

Output values

| Query out | Select (U16) | STB Returns current settings |
| --- | --- | --- |

## GL500_Status_Condition.vi

Output values

| Query out | bool array | Device status register |
| --- | --- | --- |

## GL500_Status_EESE.vi

## GL500_Status_EESR.vi

Output values

| Query out | bool array | Extended event status register |
| --- | --- | --- |

## GL500_Status_Error.vi

Output values

| Query out | I32 | Error code |
| --- | --- | --- |

## GL500_Status_Filter.vi

Input values

| bit | I32 | bit number |
| --- | --- | --- |
| Filter | Select (U16) | NEV, RISE, FALL, BOTH |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Status_Lock.vi

Input values

| Lock | Select (U16) | ON, OFF |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Status_Remote.vi

| Remote | Select (U16) | ON, OFF |
| --- | --- | --- |

## GL500_Transfer_Close.vi

Output values

| Close Error | bool | Outputs Close error |
| --- | --- | --- |

## GL500_Transfer_Data.vi

Input values

| Data | I32 | Sets number of data output points |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | Returns the current settings |
| --- | --- | --- |

## GL500_Transfer_Head.vi

Output values

| Head | STR | Header data |
| --- | --- | --- |
| Status | I32 | Receiving state |

## GL500_Transfer_ID.vi

Input values

| ID | I32 | ID number |
| --- | --- | --- |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | Returns the current settings |
| --- | --- | --- |

## GL500_Transfer_Open.vi

Output values

| ID | I16 | ID number |
| --- | --- | --- |
| Open Error | bool | Outputs Open error |

## GL500_Transfer_Source.vi

Input values

| Source | I32 | MEM, DISK |
| --- | --- | --- |
| File path | STR | File path/memory number |
| Query | bool | Returns settings status |

Output values

| Source out | I32 | Returns MEM, DISK |
| --- | --- | --- |
| File path out | STR | Returns file path/memory number |

## GL500_Trigger_Channel.vi

Input values

| Start/Stop | I32 | Start (trigger start), Stop (trigger stop) |
| --- | --- | --- |
| Channel | I32 | Channel number |
| Set | I32 | Trigger OFF, High, Low, Window Window IN/OUT, %1, %2 HI/LO HI/LO |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | Returns the current settings |
| --- | --- | --- |

## GL500_Trigger_Combination.vi

Input values

| Start/Stop | I32 | Start (trigger start), Stop (trigger stop) |
| --- | --- | --- |
| Comb | Select (U16) | OR/AND |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns OR/AND |
| --- | --- | --- |

## GL500_Trigger_Logic.vi

Input values

| Start/Stop | I32 | Start (trigger start), Stop (trigger stop) |
| --- | --- | --- |
| Channel | I32 | Channel number |
| Type | Select (U16) | OFF, HI,LO |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Trigger_Pulse.vi

Input values

| Start/Stop | I32 | Start (trigger start), Stop (trigger stop) |
| --- | --- | --- |
| Channel | I32 | Channel number |
| Set | I32 | Trigger OFF, High, Low, Window Window IN/OUT, %1, %2 HI/LO HI/LO |
| Query | bool | Returns settings status |

Output values

| Query out | I32 | Returns the current settings |
| --- | --- | --- |

## GL500_Trigger_Source.vi

Input values

| Start/Stop | I32 | Start (trigger start), Stop (trigger stop) |
| --- | --- | --- |
| Source | Select (U16) | Off, Amp, Date, Extern, Time, Event |
| Time | STR | Sets date and time when Time is specified |
| Query | bool | Returns settings status |

Output values

| Query out | Select (U16) | Returns the current settings |
| --- | --- | --- |

## GL500_Up_Execute.vi

Prepares for update data transfer.